ME400 - Computer Applications in Mechanical Engineering

Project #2

# Instructions:

Complete each of the following steps. When you have completed all steps assigned, upload the files associated with the project on Canvas.   Make sure the filenames you upload as part of your project submission have the exact names specified in each step of the project.  If the filenames do not match exactly, then your project submission will not be graded.

# Problem Description

You have been hired as a Controls Engineer at Planet Safe. You must build a prototype autonomous robot that uses an ultrasonic sensor sweep to detect an object, interface with two gear motors and two optocouplers to control and monitor robot motion, and navigate across the surface of a remote planet. The project is divided into four options that progressively develop the sensing, calibration, and navigation capabilities of the robot.

For the prototype, your team will need to assemble the following components:

- One ESP32-S3-DevKit and ME400 shim.

- Two servos.

- One pan/tilt assembly.

- One ultrasonic sensor.

- One dual-channel DRV8835 motor driver configured for PHASE/ENABLE control.

- Two optocouplers.

- Two encoder wheels.

- Two gear motors.

- Two 65-mm rubber wheels.

- One two-wheel robot chassis with a passive caster.

- One battery pack to power the esp32, motors, and encoders.

You must connect your components exactly as shown in the figure posted on canvas.  Make sure you securely connect the connectors to the ME400 shim to ensure proper connection.  The connections required for this project are shown in the figure on canvas.  These connections must be setup in your program as follows:

- Connect the PAN and TILT servos as shown.   Two three pin male connectors are provided on the ME400 shim for this purpose.  Plug the servos directly into the provided connectors with the orientation shown in the figure.

- Connect the ultrasonic sensor to the four-pin female connector on the ME400 shim.  Use four of the female-to-male jumper wires to connect the sensor to the provided connector.  Ensure that the wires are oriented correctly.

- Setup the motor driver on the breadboard, attach the two motors, and attach the associated optocouplers as shown in the figure on canvas.  Details related to the motor driver are outlined in the section below

DRV8835 Motor Driver

The DRV8835 is a dual H-bridge motor driver that allows the ESP32-S3 to independently control the speed and direction of two DC motors. For this project, the driver is operated in PHASE/ENABLE mode. The PHASE (PH) input controls motor direction, while the ENABLE (EN) input controls whether the motor is driven and can accept a PWM signal to control motor speed. When EN is LOW, the motor is electrically braked; when EN is HIGH, the PH input determines the direction of rotation.  The required connections between the ESP32-S3 to the DRV8835 driver are outlined below:

| Motor | ESP32-S3 Pin | DRV8835 Input | Purpose |
| --- | --- | --- | --- |
| Left | GPIO 12 | A-PHASE | Direction |
| Left | GPIO 46 | A-ENABLE | PWM / Speed |
| Right | GPIO 13 | B-PHASE | Direction |
| Right | GPIO 14 | B-ENABLE | PWM / Speed |

Once you have connected the components, write a program as outlined in the requirements.

# Requirements:

SPECIAL NOTE:  FOR THIS EXERCISE YOU ARE ONLY ALLOWED TO USE FUNCTIONALITY COVERED IN THE TEXTBOOK OR COVERED IN CLASS.  USE OF FUNCTIONALITY NOT COVERED IN CLASS WILL RESULT IN A ZERO BEING ASSESSED FOR THIS PROJECT!!!

- A component kit will be supplied to your group when you attend your kick off meeting with your TA.  This component kit will include a battery pack that must be returned when you complete the project.

- Ensure all connections shown in the figure on canvas have been completed before starting development.  PLEASE BE VERY CAREFUL TO MAKE SURE ALL CONNECTIONS ARE CORRECT.  IMPROPER CONNECTIONS CAN SEVERELY DAMAGE THE EQUIPMENT!

- Assemble your tilt/pan assembly based on the instructions posted on canvas.  This pan/tilt assembly must be assembled and displayed with your submission.  Mount the ultrasonic sensor to the top plate of the pan/tilt assembly so that it can be used for a radar sweep.  Attach the pan/tilt assembly to your ME400 shim using some of the extra screws and nuts provided with your pan/tilt assembly.

- Attach the ultrasonic sensor to the top plate of the pan/tilt assembly such that the ultrasonic sensor sits perpendicular to the top plate.  Design an adapter plate to attach the ultrasonic sensor to the pan/tilt assembly and 3D print it to obtain 5 points of extra credit.  To obtain the extra credit arrange a time to show the 3D component mounted to your pan/tilt assembly to your instructor (in this case Ronnie Brockhoff).  If you do not design a 3D component, then simply glue the ultrasonic sensor to the top plate of the pan/tilt assembly.

- Print the parts for the robot chassis that are posted to canvas.  Attach the optocoupler mounts to the main chassis and then attach both the optocouplers and the motors to the chassis.  Attach the ME400 shim to the top of the chassis and connect the battery pack to the USB connector on the ESP32.  WARNING: THIS WILL POWER ON THE ESP32 AND POWER THE OTHER COMPONENTS.  UNPLUG THE BATTERY PACK TO TURN OFF THE SYSTEM.  For reference if you do not already know, free 3D printing services are provided in the maker space on the 3rd floor.

- Create a sub directory named “project2a” within the Arduino subdirectory of your VSCode workspace on the Z: drive.  Download the files project2a.ino, lcdhelperv2.ino, lcdhelperv2.h, irhelper.ino, and irhelper.h from the project2 resources on canvas.  DO NOT make changes to the files lcdhelperv2.h, lcdhelperv2.ino, irhelper.h, or irhelper.ino.  Changes should only be made to the file project2a.ino that you will implement as part of this project.

- So far, you haven’t written any code.  However, if you upload the sketch in its current state, then the sketch should compile.  Go ahead and upload the current version of the sketch to your Arduino.  You should see a menu with the following options:

SELECT OPTION

1.CALIBRATE DRIVE

2.SEEK TARGET

3.LOCATE TARGET

4.NAVIGATE

- Implement code so that options 2 and 4 are disabled (and displayed in red) in the option title).  Once option 1 has been completed enable option 2 and redisplay the menu as follows:

SELECT OPTION

1.CALIBRATE DRIVE

2.SEEK TARGET

3.LOCATE TARGET

4.NAVIGATE

- Implement code so that option 4 is disabled (and displayed in red) in the option title).  Once option 1 and 3 have been completed enable option 4 and redisplay the menu as follows:

SELECT OPTION

1.CALIBRATE DRIVE

2.SEEK TARGET

3.LOCATE TARGET

4.NAVIGATE

- Now, it’s time for you to start adding support for the code you are going to write.  NOTE: No external libraries other than the lcdhelper, and irhelper libraries may be used in this project.  Use of any other library or code from the libraries will result in a zero being assigned for the portion of the project for which the library is used.

- Declare a global constant integer variable to store the digital pin (i.e., pin 8) connected to output channel of the right optocoupler.  Declare a global constant integer variable to store the digital pin (i.e., pin 10) connected to output channel of the left optocoupler. Call the pinMode() function in the setup() routine to declare the pins will be used for input in which the pins are pulled LOW when disconnected (the pulldown is performed externally using a 10K resistor on the breadboard).

- Declare four global constant integer variables to store the digital pins (i.e., pins 18, 17, 16, and 15) for the four switches (i.e., sw#1, sw#2, sw#3, and sw#4) on the ME400 shim.  Call the pinMode() function in the setup() routine to declare the pins will be used for input. These pins should also be setup such that the pin reads HIGH when disconnected and LOW when the associated button is pressed.  Note: You could also declare an array of constant integers and store the values for the four pins in the array.

- Declare two global constant integer variables to store the digital pins (i.e., pin 35 and 36) connected to the tilt and pan servos, respectively.  Call the pinMode() function in the setup() routine to declare the pins will be used for output.

- Declare two global constant integer variables to store the digital pins (i.e., pin 37 to ECHO and 38 to TRIG) connected to the echo and trigger pins of the ultrasonic sensor.  Call the pinMode() function in the setup() routine to declare the echo will be used for input and the trigger pin will be used for output.

- Declare one global constant integer variable to store the digital pin (i.e., pin 46) for the ENABLE pin of the left motor. Declare one global constant integer variable to store the digital pin (i.e., pin 14) for the ENABLE pin of the right motor.   Call the pinMode() function in the setup() routine to declare the pin will be used for output.

- Declare one global constant integer variable to store the digital pin (i.e., pin 12) for the PHASE pin of the left motor. Declare one global constant integer variable to store the digital pin (i.e., pin 13) for the PHASE pin of the rght motor.   Call the pinMode() function in the setup() routine to declare the pin will be used for output.

- Declare a global constant integer to represent the PWM channel for the left motor and set the value to 1.

- Declare a global constant integer to represent the PWM channel for the right motor and set the value to 2.

- Implement code in the setup() routine to setup a connection to the serial port at a speed of 115,200 bits/second. Within setup() implement code to perform the following:

  - Set the tilt servo to the 15-degree position and the pan servo to the 90-degree position.  Adjust your pan/tilt assembly so that this positions the platform level with the ground and in the center of the extremes for the swing of the pan servo.  This will involve using the provided servowrite() function.  More detail about the use of this function is provided in the documentation for Option 3.

  - Assign the ENABLE pin for the left motor with the channel created in Step 17 and assign the ENABLE pin for the right motor with the channel created in Step 18.

  - Implement code to set the duty cycle for the signal on the ENABLE pin for both motors to zero.  This will ensure the motors are not moving when the program first starts.

- For Option 1 refer to Project1Option1.docx to determine the requirements for CALIBRATE DRIVE.

- For Option 2 refer to Project1Option2.docx to determine the requirements for SEEK TARGET.

- For Option 3 refer to Project1Option3.docx to determine the requirements for LOCATE TARGET.

- For Option 4 refer to Project1Option4.docx to determine the requirements for NAVIGATE TO TARGET.

- Your team is must schedule a meeting on or before October 16th with a ME400 TA.  A team leader should be appointed for every team during this meeting.  All team members must attend the meeting.  A 10-point deduction will be imposed on any team that does not schedule a meeting.  Furthermore, every team member that does not attend this meeting will receive a 10-point deduction.

- Your team is must schedule a followup meeting on or before October 28th with a ME400 TA.  The team must demonstrate that the robot is fully assembled, that all wiring is connected appropriate, and that at minimum code has been loaded to the associated esp32 and menus are displaying corectly.  A 10-point deduction will be imposed on any team that does not schedule a meeting.  Furthermore, every team member that does not attend this meeting will receive a 10-point deduction.

- Your team is expected to create a one-page workload assessment document that includes the team’s estimate of the percentage of the overall work each of the following tasks represents (Percentages must total to 100%):

- Task 1: Hardware (i.e., wiring, 3D printing, physical assembly, etc).

- Task 2: Completion of main program including changes to key handling, enabling of menus, and communicating data between options.

- Task 3: Project collaboration including setting up meetings.

- Task 4: Completion of Option 1

- Task 5: Completion of Option 2

- Task 6: Completion of Option 3

- Task 7: Completion of Option 4

- Task 8: Demonstration Preparation and Troubleshooting

A printed copy of this assessment must be turned induring the lecture at 1:30pm on Wednesday November 11th.

- Each team member must individually fill out the project workload actuals survey on canvas.  You must enter the percentage of each of the eight tasks in the workload assessment document that you completed on a scale of 0 to 100%.  For a group the total for all team members for each task should ideally be 100%.  However, if these totals are not 100% (or close to 100%), then your team will be expected to meet with your instructor to explain the reason for the discrepancy.  Your team will receive an overall score based on the overall functionality of the associated project submission.  If one team member does not complete a task, then it is up to the rest of the team to complete the task.  However, the team member that did not complete the assigned task will receive a significant deduction based on the workload assessment and workload actuals survey.

- This project will involve a project competition that will be performed during your team’s assigned lab period on Friday November 13th.  All team members must be present for the demonstration.  Any team member that is absent will be assessed a 10-point penalty.  The lab instructor assigned to your lab section will have you demonstrate your function.  The performance of each robot will be ranked and part of your group’s final grade will be based on this ranking.

# Program Submission:

Upload the file project2a.ino to canvas

Your submission will be evaluated as follows:

- They will be evaluated to make sure the proper coding techniques were used to implement the associated functionality.

- They will be evaluated to ensure the associated code compiles successfully.

- They will be evaluated to ensure the associated code returns accurate and complete results.

- They will be evaluated to determine if sufficient comments were entered to discern the intended functionality.

- They will be evaluated to determine if the code was completed independently without use of external code.

- They will be evaluated to ensure that the correct file names specified for each problem were uploaded to canvas.

- Your code will be evaluated to make sure the implementation uses correct indentation.  To ensure proper indentation, select any of the code you added, right click in VSCode, and select Format Selection.