ME400 - Computer Applications in Mechanical Engineering

# Project #2 Option 1

# Instructions:

Complete each of the following steps. When you have completed all steps assigned, upload the files associated with the project on Canvas.   Make sure the filenames you upload as part of your project submission have the exact names specified in each step of the project.  If the filenames do not match exactly, then your project submission will not be graded.

# Option 1 Requirements

In this option, you will develop and test a straight-line drive control mechanism that will be used by the robot in later portions of the project. Even when identical duty cycles are applied to the two motors in your kit, differences in the motor performance, gearbox inconsistencies, wheel loading, and battery conditions can cause the two wheels to travel different distances in the same timer interval. To correct for this behavior, the encoder attached to each motor will be used to monitor the distance the wheel travels and correct for this difference in real time while the robot is moving.

Your task for this option is to allow the user to select a base duty cycle and then command the robot to travel 100 centimeters in a straight line. While the robot is moving, your program must monitor the encoder count for each wheel and use the difference between the two counts to keep the wheel travel approximately equal. Your program must satisfy the requirements listed below (Note: You are not allowed to use any third-party libraries to complete this task.  You must complete the task using code that your group generates):

- When Option 1 is selected immediately update the screen as follows:

DUTY CYCLE

000

PRESS

<RETURN>

WHEN

READY

- Implement code to allow the desired duty cycle to be entered using the IR remote.  Only numerical values should be displayed.  An example is shown below:

DUTY CYCLE

0600

PRESS

<RETURN>

WHEN

READY

- For step 2 implement code such that once a number is enterred pressing the back (i.e., rewind) key on the remote will delete the last digit of the number enterred.  For example if the number 100 were enterred, pressing the back key once should redisplay the number as 10.  Pressing the back key again should redisplay the number as 1, and pressing the back key again should redisplay the entry as blank.  This is an imporant requirement.  It is common to mistrike keys during entry and without this functionality the user is stuck with the incorrect entry and forced to drive the motor at a duty cycle that is not desired.

For help with the type of key handling used in this option refer to the code you worked with in exercises #5.  This exercise invovled enterring parameters for the minor and major axis of an ellipse and calculating the area.  The key handling for this exercise is very similar to that required in this step.

- For step2 implement checks on the data that only allows the entry of a duty cycle between 40% and 70%.  In other words, do not accept a key entry in the following situations:

  - Any duty cyle between 0 and 39 should be rejected.  Therefore any number starting with 0, 1, 2, or 3 should be rejected since no combination start with this digit is valid.

  - If the first digit enterred is 4 through 6, then the number should be limited to 2 digits (i.e., limited to numbers between 40 and 69).  If the first digit is 7, then the second digit should be limited to 0.

- Once the user enters the desired duty cycle and presses the return key (double array key), process the entry as follows:

  - If the duty cycle is less than 40% then the return key should be ignored and the program should wait for additional input.  In other words, if the user enterred a duty cycle of 4 then the user should not be able to hit return.  However, if they go ahead and add a second digit, then the duty cycle would be valid and they should be able to press return.

  - Reset the encoder count for both wheels to zero and prepare the robot to travel 100 centimeters. The entered duty cycle represents the base duty cycle for the straight-line motion.

  - Determine the target number of encoder counts, NTARGET, corresponding to 100 centimeters of wheel travel. The wheel diameter and the number of encoder counts per revolution must be used to perform this calculation. Do not hard-code the target encoder count without showing the calculation in your program comments.

  - Command both motors to move the robot forward. Use the entered duty cycle as the initial PWM duty cycle for both motors. The motor direction must be controlled using the PHASE inputs of the DRV8835 motor driver and the motor speed must be controlled using the ENABLE inputs of the DRV8835 motor driver.

  - While the robot is moving, continually monitor the accumulated encoder count for each wheel. Define the encoder error as:

ERROR = LEFT ENCODER COUNT - RIGHT ENCODER COUNT

A positive error indicates that the left wheel has traveled farther than the right wheel. A negative error indicates that the right wheel has traveled farther than the left wheel.

  - Use the encoder error to adjust the duty cycle of the right wheel while the robot is moving so that the two encoder counts remain approximately equal. Your program must make the correction automatically (i.e., the user may not manually adjust the duty cycle after the 100-centimeter test begins). In addition, the correction should be based on the entered duty cycle and should not command the right motor outside the range of 20% to 80% duty-cycle range.  Your group must develop an algorithm that determines when a correction is required and the amount by which the right motor duty cycle should be adjusted.

Note: The left motor must remain at the user-entered duty cycle throughout the 100-centimeter test. Only the right motor duty cycle may be adjusted to correct encoder error.

  - Every 1 seconds, update the TFT to display the commanded duty cycle on the right wheel, the accumulated encoder counts and the current encoder error. The display should have the following general format:

RIGHT DUTY CYCLE

67.6

LEFT COUNT

00000

RIGHT COUNT

00000

ERROR

+000

Note: the value 00000 for the left and right motors and the value +000 should be replaced with the actual numeric values found in your program

Note: The one-second interval applies only to updating the TFT. Encoder monitoring and motor correction must occur continuously while the robot is moving.

  - Continue monitoring the encoder counts until the robot has traveled 100 centimeters or until the user presses the power button the remote.  If the user presses the power button on the remote then both motors should be command to stop and the screen should updated to show the current duty cycle for the right motor, the counts for both motors and the error.  . Otherwise, determine the accumulated encoder counts measured for the left and right wheels NL and NR. The robot should be considered to have traveled 100 centimeters when the encoder count on both wheels is greater than or equal to the number of encoder counts calculated in Step 5c for 100 centimeters of wheel travel:

NL≥NTARGET and NR≥NTARGET

Immediately stop both motors when this condition is satisfied and update the screen as follows:

RIGHT DUTY CYCLE

76.7

LEFT COUNT

00000

RIGHT COUNT

00000

ERROR

+000

Note: the value 00000 for the left and right motors and the value +000 should be replaced with the actual numeric values found in your program

  - The final encoder counts should be approximately equal and the error should also be approximately zero. The quality of the straight-line control will be evaluated based on the final encoder error, the measured distance traveled by the robot, and the lateral deviation of the robot from a straight path.

  - Cache any values or parameters developed as part of the straight-line drive control in global variables such that they can be used in to characterize the motor speed for the other project options. The straight-line drive functionality developed in this option should be implemented so that it can be reused by later options rather than duplicated.

- Implement code so that pressing the the mode button on the remote returns to the main menu.  This option should be available as long as the duty cycle has not already been enterred or after the robot has travelled 100 centimeters.  If the return button is pressed, then Step 5 should be repeated for the same duty cycle that was initially enterred.  This allows the user to run the robot an additional 100 centimeters and continue the analysis of the motor performance.  Note: the user should not be allowed to return to the main menu while the robot is in motion.