ME400 - Computer Applications in Mechanical Engineering

# Project #2 Option 4

# Instructions:

Complete each of the following steps. When you have completed all steps assigned, upload the files associated with the project on Canvas.   Make sure the filenames you upload as part of your project submission have the exact names specified in each step of the project.  If the filenames do not match exactly, then your project submission will not be graded.

# Option Requirements:

At this point, Option 3 has determined and stored the direction and distance to the target. Before Option 4 is selected, the target will be removed. The robot must then navigate to the stored target location without using the ultrasonic sensor to locate or correct its path.

For Option 4 implement code that seeks the target.  The details associated with this implementation are listed below:

- Implement code so that Option 4 cannot be selected until Option 1 calibration has been completed and Option 3 has successfully stored a valid target direction and distance.

- Do not move or rotate the robot between completion of Option 3 and selection of Option 4. Only the target should be removed (and this step will be performed by your instructor during the live demonstration).

- The existing code calls a function Option4() when Option 4 is executed.  Implement your code within this function.  Note:  You are highly encouraged to break the code up for this option into several functions and call this code from your implementation in the function Option4().  Placing all of your functionality in Option4() without the use of supporting functions is considered bad coding practice and you will receive a deduction if you implement your code in this matter.

- When Option 4 is selected, immediately display the stored TARGET DIRECTION and TARGET DISTANCE on the TFT in portrait orientation as shown below:

TARGET DIR

27.5 DEGREES

TARGET DIST

57.5 CM

Note:  The angle and distance displayed should match the value obtained when Option 3 was executed.

- After Option 4 begins execution, the ultrasonic sensor may not be used to determine the target location or correct the robot path. Navigation must be based on the target information stored by Option 3 and the wheel encoders.  Technically since your instructor will remove the target before execution of this step any data returned by the ultrasonic sensor would not be useful for this step.

- Move the robot forward such that the midpoint between the two drive wheels is directly over the original pivot point of the pan/tilt assembly. It is up to you to determine the appropriate travel distance required for this step. Perform the movement at a duty cycle of 30%.

- Pause execution of your code so that the instructor can record the values displayed on your TFT screen.  Implement the code to continue execution when the sw#1 is pressed on the ME400 shim.

- Once the midpoint between the two drive wheels is situated over the original pivot point of the pan/tilt assembly, perform an in-place (zero-radius) turn so that the robot is pointed toward the stored target direction obtained in Option 3. Assume the robot's starting direction is 90 degrees. Therefore, the required angular rotation is the difference between the stored target direction and 90 degrees. Use the known wheel spacing, wheel diameter, and encoder measurements to determine the required wheel travel for the commanded rotation.

Note: The encoders provide relatively coarse angular resolution, so the encoder counts alone may not place the robot exactly at the desired target angle. Performance may be improved by using the measured wheel speed to estimate the additional motor run time required for the fractional encoder count.

- During the rotation, monitor both wheel encoders and stop each motor when its calculated wheel travel for the commanded rotation has been achieved. Update the TFT display to show the calculated current direction:

TARGET DIR

27.5 DEGREES

TARGET DIST

57.5 CM

CURRENT DIR

27.0 DEGREES

- Pause execution of your code so that the instructor can record the values displayed on your TFT screen and verify the physical location of your robot.  Implement the code to continue execution when the sw#2 is pressed on the ME400 shim.

- After the robot is pointed toward the stored target location, drive the robot forward through the stored travel distance obtained when Option 3 was executed. Use the duty cycle selected during Option 1 as the base duty cycle for this movement.

- Use the wheel diameter and encoder counts per revolution to convert encoder measurements to linear travel distance.

- Compare the left and right encoder measurements while the robot moves forward. Adjust the commanded motor speeds as necessary to reduce differences in wheel travel and maintain a straight path.

- Use the motor-calibration information generated in Option 1 when determining the PWM commands for the two motors.

- Stop both motors when the calculated travel distance has been reached. Update the TFT display and display the distance traveled as outlined below:.

TARGET DIR

27.5 DEGREES

TARGET DIST

57.5 CM

CURRENT DIR

27.0 DEGREES

CURRENT DIST

58.0 CM

- Pause execution of your code so that the instructor can record the values displayed on your TFT screen and verify the physical position of the robot.  Implement the code to continue execution when the sw#3 is pressed on the ME400 shim.  Technically pressing the button will not update the display, but will prevent an accidental return to the main menu if the return key is pressed before your instructor is ready for you to move on.

- Pressing the return key on the IR remote should return to the main menu.