LCDHELPER

Object for Interfacing with TFT Display

# MANUAL

PREAMBLE

The lcdhelper class is a wrapper that inherits from the UTFT class.  For detailed document of the underlying UTFT class, refer to the document “UTFT, Multi-Platform Universal TFT display library”, which is maintained by Rinky-Dink Electronics.  To use the associated wrapper class , the files lcdhelper.h and lcdhelper.cpp must be included in the same directory as the Arduino sketch file.  In addition, a reference to header file must be included in the main sketch file as shown below:

#include "lcdhelper.h"

To use the lcdhelper class to display of data on the TFT display, an instance of the lcdhelper class must be declared.  This declaration calls the default constructor of the lcdhelper class.  A detailed description of the constructor is as follows:

lcdhelper(Model, RS, WR, CS, RST)

Description

Defines the main class constructor when using 8 bit or 16 bit display modules.

Parameters

Model: See the separate document for the supported display modules RS: Pin for Register Select WR: Pin for Write CS: Pin for Chip Select RST: Pin for Reset ALE: Pin for Latch (optional) Only used for latched 16bit shields)

Usage:

/ // Create an instance of the lcdhelper class named lcd // lcdhelper oLCD(ILI9163_4L,3,2,9,10,7);

SUPPORTED FUNCTIONS

This section provides details related to the functions supported in the lcdhelper class.  The associated example usage is based on declaring an object named oLCD.

LCDInitialize(orientation)

Description

Initializes the display with PORTRAIT or LANDSCAPE orientation, sets the font to the small font, clears the screen, sets the background to purple, creates a border around the edge of the screen, and sets the font color to white.

Parameters

orientation: PORTRAIT or LANDSCAPE

Usage:

// // Initialize the display // oLCD.LCDInitialize(PORTRAIT);

InitLCD([orientation])

Description

Initialize the LCD and set display orientation.

Parameters

orientation: PORTRAIT or LANDSCAPE (default)

Usage:

// // Initialize the display. Note: This will reset color to white with black // background. Selected font will be reset to none. // oLCD.InitLCD(PORTRAIT);

getDisplayXSize()

Description

Get the width of the screen in the current orientation in pixels.

Parameters

None

Usage:

// // Get screen width in pixels // int x = oLCD.getDisplayXSize();

getDisplayYSize()

Description

Get the height of the screen in the current orientation in pixels.

Parameters

None

Usage:

// // Get screen height in pixels // int x = oLCD.getDisplayYSize();

lcdOff()

Description

Turn off the LCD. No commands will be executed until a lcdOn(); is sent. Note: Only supported on PCF8833 and CPLD-based displays.

Parameters

None

Usage:

oLCD. lcdOff();

lcdOn()

Description

Turn on the LCD after issuing a lcdOff()-command Note: Only supported on PCF8833 and CPLD-based displays.

Parameters

None

Usage:

oLCD. lcdOn();

setContrast(c);

Description

Set the contrast of the display. Note: This function is currently only supported on PCF8833-based displays.

Parameters

c: Contrast-level (0-64)

Usage:

// // Set contrast to full // oLCD. setContrast(64);

setBrightness(br)

Description

Set the brightness of the display backlight. Note: This function is currently only supported on CPLD-based displays

Parameters

br: Brightness-level (0-16)

Usage:

// // Set brightness to maximum // oLCD. setBrightness(16);

setDisplayPage(pg)

Description

Set which memory page to display. Note: This function is currently only supported on CPLD-based displays

Parameters

pg: Page (0-7) (0 is default)

Usage:

// // Display content on memory page 4. // oLCD. setDisplayPage(4);

setWritePage(pg)

Description

Set which memory page to use for subsequent display writes. Note: This function is currently only supported on CPLD-based displays

Parameters

pg: Page (0-7) (0 is default)

Usage:

// // Write subsequent data to page 2. // oLCD. setWritePage(2);

clrScr()

Description

Clear the screen. The background-color will be set to black.

Parameters

None

Usage:

// // Clear the screen // oLCD. clrScr();

fillScr(r, g, b)

Description

Fill the screen with a specified color.

Parameters

r: Red component of an RGB value (0-255) g: Green component of an RGB value (0-255) b: Black component of an RGB value (0-255)

Usage:

// // Fill screen with orange. // oLCD. fillScr(255,127,0);

fillScr(color)

Description

Fill the screen with a specified pre-calculated RGB565 color.

Parameters

color: RGB565 color value

Usage:

// // Fill screen with purple // oLCD. fillScr(VGA_PURPLE);

setColor(r, g, b);

Description

Set the color to use for all draw*, fill* and print commands.

Parameters

r: Red component of an RGB value (0-255) g: Green component of an RGB value (0-255) b: Black component of an RGB value (0-255)

Usage:

// // Set color to orange. // oLCD. setColor(255,127,0);

setColor(color)

Description

Set the specified pre-calculated RGB565 color to use for all draw*, fill* and print commands.

Parameters

color: RGB565 color value

Usage:

// // Set color to aqua. // oLCD. setColor(VGA_AQUA);

getColor()

Description

Get the currently selected color.

Parameters

None

Usage:

// // Returns currently selected color as a RGB565 value (word) // word Color = oLCD.getColor();

setBackColor(r, g, b)

Description

Set the background color to use for all print commands.

Parameters

r: Red component of an RGB value (0-255) g: Green component of an RGB value (0-255) b: Black component of an RGB value (0-255)

Usage:

// // Set background color to white // oLCD.setBackColor(255,255,255);

setBackColor(color)

Description

Set the specified pre-calculated RGB565 background color to use for all print commands.

Parameters

color: RGB565 color value

Usage:

// // Set color to VGA_LIME // oLCD.setBackColor(VGA_LIME);

getBackColor()

Description

Get the currently selected background color.

Parameters

None

Usage:

// // Returns currently selected background color as a RGB565 value (word) // word color = oLCD.getBackColor();

drawPixel(x, y)

Description

Draw a single pixel

Parameters

x: x-coordinate of the pixel y: y-coordinate of the pixel

Usage:

// // Draw a single pixel at coordinate 119, 159 // oLCD.drawPixel(119,159);

drawLine(x1, y1, x2, y2)

Description

Draw a line between two points.

Parameters

x1: x-coordinate of the start-point y1: y-coordinate of the start-point x2: x-coordinate of the end-point y2: y-coordinate of the end-point

Usage:

// // Draw a diagonal line // oLCD.drawLine(0,0,239,319);

drawRect(x1, y1, x2, y2)

Description

Draw a rectangle between two points

Parameters

x1: x-coordinate of the start-corner y1: y-coordinate of the start-corner x2: x-coordinate of the end-corner y2: y-coordinate of the end-corner

Usage:

// // Draw a rectangle // oLCD.drawRect(119,159,239,319);

drawRoundRect(x1, y1, x2, y2)

Description

Draw a rectangle with slightly rounded corners between two points. The minimum size is 5 pixels in both directions. If a smaller size is requested the rectangle will not be drawn.

Parameters

x1: x-coordinate of the start-corner y1: y-coordinate of the start-corner x2: x-coordinate of the end-corner y2: y-coordinate of the end-corner

Usage:

// // Draw a rounded rectangle // oLCD.drawRoundRect(0,0,119,159);

fillRect(x1, y1, x2, y2)

Description

Draw a filled rectangle between two points.

Parameters

x1: x-coordinate of the start-corner y1: y-coordinate of the start-corner x2: x-coordinate of the end-corner y2: y-coordinate of the end-corner

Usage:

// // Draw a filled rectangle // oLCD.fillRect(119,0,239,159);

fillRoundRect(x1, y1, x2, y2)

Description

Draw a filled rectangle with slightly rounded corners between two points. The minimum size is 5 pixels in both directions. If a smaller size is requested the rectangle will not be drawn.

Parameters

x1: x-coordinate of the start-corner y1: y-coordinate of the start-corner x2: x-coordinate of the end-corner y2: y-coordinate of the end-corner

Usage:

// // Draw a filled rounded rectangle // oLCD.fillRoundRect(0,159,119,319);

drawCircle(x, y, radius)

Description

Draw a circle with a specified radius.

Parameters

x: x-coordinate of the center of the circle y: y-coordinate of the center of the circle radius: radius of the circle in pixels

Usage:

// // Draw a circle with a radius of 20 pixels // oLCD.drawCircle(119,159,20);

fillCircle(x, y, radius)

Description

Draw a filled circle with a specified radius.

Parameters

x: x-coordinate of the center of the circle y: y-coordinate of the center of the circle radius: radius of the circle in pixels

Usage:

// // Draw a filled circle with a radius of 10 pixels // oLCD.fillCircle(119,159,10);

print(st, x, y[, deg])

Description

Print a string at the specified coordinates. You can use the literals LEFT, CENTER and RIGHT as the x-coordinate to align the string on the screen.

Special Notes

Note #1: CENTER and RIGHT will not calculate the coordinates correctly when rotating text. Note #2: The string can be either a char array or a String object

Parameters

st: the string to print x: x-coordinate of the upper, left corner of the first character y: y-coordinate of the upper, left corner of the first character deg: (optional) Degrees to rotate text (0-359). Text is rotated around the upper left corner.

Usage:

// // Print “Hello World!”. // oLCD.print(F("Hello, World!"),CENTER,0);

printNumI(num, x, y[, length[, filler]])

Description

Print an integer number at the specified coordinates. You can use the literals LEFT, CENTER and RIGHT as the x-coordinate to align the string on the screen.

Parameters

num: the value to print (-2,147,483,648 to 2,147,483,647) INTEGERS ONLY x: x-coordinate of the upper, left corner of the first digit/sign y: y-coordinate of the upper, left corner of the first digit/sign length: (optional) minimum number of digits/characters (including sign) to display filler filler: (optional) filler character to use to get the minimum length. The character will be inserted in front of the number, but after the sign. Default is ' ' (space).

Usage:

// // Display the value stored in the integer variable num // oLCD.printNumI(num,CENTER,0);

printNumF(num, dec, x, y[, divider[, length[, filler]]])

Description

Print a floating-point number at the specified coordinates. You can use the literals LEFT, CENTER and RIGHT as the x-coordinate to align the string on the screen.

Parameters

num: the value to print dec: digits in the fractional part (1-5) 0 is not supported. Use printNumI() instead. x: x-coordinate of the upper, left corner of the first digit/sign y: y-coordinate of the upper, left corner of the first digit/sign divider: (optional) Single character to use as decimal point. Default is '.' length: (optional) minimum number of digits/characters (including sign) to display filler filler: (optional) filler character to use to get the minimum length. The character will be inserted in front of the number, but after the sign. Default is ' ' (space).

Usage:

// Display the value stored in the float variable num with 3 fractional digits oLCD.printNumF(num, 3, CENTER,0);

setFont(fontname)

Description

Select font to use with print(), printNumI() and printNumF().

Special Notes

You must declare the font-array as an external or include it in your sketch.

Parameters

fontname: Name of the array containing the font you wish to use

Usage:

// // Set font to BigFont // oLCD.setFont(BigFont);

getFont()

Description

Get the currently selected font

Parameters

fontname: Name of the array containing the font you wish to use

Usage:

// // Get the currently selected font. // uint8_t *currentfont = oLCD.getFont();

getFontXsize()

Description

Get the width of the currently selected font in pixels.

Parameters

None

Usage:

// // Get font width // int width = oLCD.getFontXsize();

getFontYsize()

Description

Get the height of the currently selected font in pixels.

Parameters

None

Usage:

// // Get font height // int height = oLCD.getFontYsize();

drawBitmap (x, y, sx, sy, data[, scale])

Description

Draw a bitmap on the screen.

Special Notes

You can use the online-tool “ImageConverter 565” or “ImageConverter565.exe” in the Tools-folder to convert pictures into compatible arrays. The online-tool is also available.

Special Requirements

Requires #include <avr/pgmspace.h> when using an Arduino other than Arduino Due

Parameters

x: x-coordinate of the upper, left corner of the bitmap y: y-coordinate of the upper, left corner of the bitmap sx: width of the bitmap in pixels sy: height of the bitmap in pixels data: array containing the bitmap-data scale: (optional) Scaling factor. Each pixel in the bitmap will be drawn as <scale>x<scale> pixels on screen

Usage:

// // Draw a 32 X 32 pixel bitmap // oLCD.drawBitmap(0, 0, 32, 32, bitmap);

drawBitmap (x, y, sx, sy, data, deg, rox, roy);

Description

Draw a bitmap on the screen with rotation.

Special Notes

You can use the online-tool “ImageConverter 565” or “ImageConverter565.exe” in the Tools-folder to convert pictures into compatible arrays. The online-tool is also available.

Special Requirements

Requires #include <avr/pgmspace.h> when using an Arduino other than Arduino Due

Parameters

x: x-coordinate of the upper, left corner of the bitmap y: y-coordinate of the upper, left corner of the bitmap sx: width of the bitmap in pixels sy: height of the bitmap in pixels data: array containing the bitmap-data deg: Degrees to rotate bitmap (0-359) rox: x-coordinate of the pixel to use as rotational center relative to bitmaps upper left corner roy: y-coordinate of the pixel to use as rotational center relative to bitmaps upper left corner

Usage:

// // Draw a bitmap rotated 45 degrees around its center // oLCD.drawBitmap(50, 50, 32, 32, bitmap, 45, 16, 16);

SUPPORTED FONTS

SmallFont

Character size:

Number of characters:

8

x12 pixels

95

BigFont

Character size:

Number of characters:

16

x16 pixels

95

SevenSegNumFont

Character size:

Number of characters:

32

x50 pixels

10